BLACKCAT ROBOTICS — FIELD OPERATIONS Certified Field Technician Program eBike · Scooter · Micromobility Fleet Services

Issued: March 2026  ·  Version 1.0  ·  BCR-CERT-EBIKE-001

Program Overview

Field Tech
Certification

The BCR Field Technician Certification qualifies technicians to inspect, diagnose, and service eBike, scooter, and micromobility platforms monitored by TechMedix. Certified techs are eligible for dispatch through the BlackCat technician marketplace and carry BCR-authorized service credentials.

This document covers: required toolkit, failure signatures, maintenance intervals, platform-specific protocols, and certification requirements for all three active micromobility platforms — Lime Gen 4, Bird Three, and RadCommercial.

Section 1 — Required Field Toolkit

Every BCR-certified eBike/Scooter technician must carry the complete toolkit below. Items marked with an asterisk are mandatory before dispatch eligibility. Total kit cost: ~$650.

| Tool | Cost | Category | Purpose |
| --- | --- | --- | --- |
| SkyRC MC3000 Analyzer | $80 | Battery | Cell-level capacity, voltage drift, cycle count per cell |
| Fluke 376 Clamp Meter | $300 | Electrical | Hub motor current draw vs load signature, ESC health |
| ADXL345 + RPi Zero Kit | $40 | Vibration | Bearing wear FFT analysis, motor vibration signature |
| Torque Wrench Set 2-25Nm | $120 | Mechanical | Stem fold torque, fork bolts, wheel axle, handlebar clamp |
| Digital Pressure Gauge | $20 | Tires | Tire pressure trending, pneumatic seal integrity |
| Wheel Truing Stand | $60 | Mechanical | Rim wobble detection, spoke tension, lateral runout |
| Dye Penetrant Kit | $30 | Structural | Frame crack detection at stem, fork crown, weld joints |
| Laptop + OEM SDK Access | varies | Software | Fault code read, firmware version, OTA status, geofence check |
| TOTAL | ~$650 |  | Complete BCR-certified field kit |

SDK access requirements by platform:

- Lime Gen 4 — Lime DASH operator portal (requires fleet operator agreement)

- Bird Three — Bird Ops Console (requires fleet operator agreement)

- RadCommercial — Rad Power Bikes Fleet API (open registration)

- Independent fleets — contact BCR for platform-specific onboarding

Section 2 — TechMedix Failure Signatures

The following failure signatures are configured in TechMedix for all three micromobility platforms. When a signature triggers, TechMedix alerts the operator and — if severity is CRITICAL — auto-queues a BCR tech dispatch.

All Platforms

| Signal ID | Threshold | Severity | Action |
| --- | --- | --- | --- |
| battery-cell-drift | ±50mV cell delta | WARNING | Schedule battery swap within 14 days. Log cycle count. |
| battery-thermal | >45°C at rest | CRITICAL | Pull from service immediately. Thermal runaway risk. |
| battery-degraded | <70% original capacity | WARNING | Flag for replacement on next scheduled service. |
| hub-motor-bearing | Vibration >0.8g RMS | WARNING | Inspect bearing, schedule motor service within 7 days. |
| hub-motor-draw | Current >15% above base | WARNING | Motor winding or controller fault. Bench test required. |
| stem-fold-wear | Torque <8Nm | CRITICAL | Ground unit. Stem failure is highest injury liability. |
| brake-pad-thin | <2mm estimated | WARNING | Schedule brake service within 30 days or 500 rides. |
| connectivity-loss | >15min offline | INFO | Check cellular module, SIM, antenna connection. |
| tire-pressure-drop | >8 PSI/day loss | WARNING | Inspect for puncture, valve seal, rim tape integrity. |
| firmware-drift | Version delta >2 builds | INFO | Queue OTA update on next connection window. |

Section 3 — Maintenance Intervals

TechMedix tracks all intervals automatically per unit. Alerts fire when a unit approaches the threshold. BCR techs are dispatched when the operator approves the maintenance queue item.

| Service Item | Interval | Platform | TechMedix Signal |
| --- | --- | --- | --- |
| Battery swap / capacity test | 500 cycles or 6 months | All | battery-degraded trigger |
| Brake inspection + pad measure | 30 days or 500 rides | All | brake-pad-thin trigger |
| Tire pressure check | 14 days | All | tire-pressure-drop trigger |
| Stem fold torque check | 90 days | Lime · Bird | stem-fold-wear trigger |
| Hub motor current baseline | 1,000 rides | All | hub-motor-draw trigger |
| Bearing vibration scan | 6 months | All | hub-motor-bearing trigger |
| Frame crack inspection | 12 months or post-crash | All | Visual + dye penetrant |
| Firmware audit | Monthly | All | firmware-drift trigger |
| Full platform bench test | Annual | All | Full TechMedix diagnostic run |
| Wheel truing | 3 months or 300 rides | All | Spoke tension, rim runout |

Section 4 — Platform-Specific Protocols

Lime Gen 4 eBike

- Battery: Swappable. Test removed batteries within 24hrs of swap using MC3000. Log SOH to TechMedix node record.

- Motor: 250W rear hub. Baseline current draw at 15km/h flat surface = 4.2–5.8A. Flag if >7A.

- IoT: Lime uses proprietary cellular module. Connectivity loss >15min triggers TechMedix alert. Check SIM tray and antenna before ordering replacement module.

- Geofence: Verify active geofence zone in DASH portal after any firmware update. Miscalibrated geofence = regulatory violation.

- Stem: 3-point folding stem with 10Nm spec. Inspect locking collar O-ring on every service visit.

Bird Three Scooter

- Battery: Integrated 36V 10.5Ah pack. Non-swappable. Full charge cycle time 4–5hrs. Thermal cutoff at 60°C — check BMS fault register in Bird Ops Console.

- Motor: 350W rear hub. Higher current draw than Lime — baseline 5.5–7A at 20km/h. Flag >10A.

- Deck: Bird Three deck has integrated alarm. Confirm alarm arm/disarm function on every service. False alarm drain can deplete battery 15% overnight.

- Brake: Electronic front brake + rear foot brake. Inspect brake lever pivot pin for corrosion — common failure in wet climates.

- Lifespan: Designed for 3x consumer scooter lifespan. Track cumulative mileage in TechMedix — flag units exceeding 8,000km for full teardown inspection.

RadCommercial eBike

- Motor: 750W Rad rear hub. Highest power draw of the three platforms. Baseline 7–9A at 25km/h. Flag >13A sustained.

- Battery: 48V 14Ah (672Wh) semi-integrated. Lockable. Test with MC3000 quarterly. Cell drift >60mV across pack = replace.

- Payload: 120 lb payload capacity. Inspect rear rack mounting bolts and frame welds at every service — high payload + road vibration accelerates fatigue.

- Range: 45+ mile range claim. If operator reports <35 miles per charge, run full battery capacity test before any other diagnosis.

- SDK: Rad Fleet API is the most open of the three platforms. TechMedix can pull ride data, battery SOC history, and fault logs directly via API key — no portal required.

Section 5 — Certification Requirements

To achieve BCR Field Technician — Micromobility certification and gain dispatch eligibility on the TechMedix marketplace, candidates must complete all of the following:

Knowledge Requirements

- Pass BCR eBike/Scooter technical exam (score ≥ 85%)  — Online via TechMedix portal

- Demonstrate battery analyzer operation on live unit  — Recorded video submission

- Demonstrate hub motor current baseline procedure  — Recorded video submission

- Complete stem torque inspection on Lime Gen 4 or Bird Three  — In-person or recorded

- Complete frame crack inspection using dye penetrant kit  — In-person or recorded

Equipment Requirements

- Own or have access to complete BCR toolkit (Section 1)  — Receipt or photo verification

- Laptop with internet access for SDK/portal access  — Required for dispatch

- Smartphone with TechMedix tech app installed  — iOS or Android

Platform Access Requirements

- TechMedix tech account created and verified

- Rad Fleet API key registered (minimum)  — Free registration at radpowerbikes.com/fleet

- Bird or Lime operator agreement (optional but preferred)  — Expands dispatch eligibility

Ongoing Requirements

- Complete 1 dispatch per 90 days to maintain active status

- Submit service report within 24hrs of each dispatch via TechMedix

- Annual re-certification exam for new platform versions

Section 6 — BCR Tech Rate Card

Certified BCR techs set their own rates within the following BCR-recommended ranges. BCR takes a 15% platform fee per dispatched job. Operators pay through TechMedix — techs receive payment within 48hrs of service report approval.

| Service Type | Rate Range | Notes |
| --- | --- | --- |
| Basic inspection + report | $45–$75 | No parts. TechMedix diagnostic run included. |
| Battery swap + test | $60–$90 | Includes MC3000 test and SOH log entry. |
| Brake service | $50–$80 | Pad replacement + lever adjustment. |
| Motor current baseline | $55–$85 | Clamp meter test + TechMedix baseline update. |
| Stem torque inspection | $40–$60 | Torque check + O-ring inspection. |
| Frame crack inspection | $65–$95 | Dye penetrant procedure + photo report. |
| Full platform service | $150–$220 | All above combined. Most common fleet contract item. |
| Emergency dispatch (<2hr) | +$50 surcharge | Operator-triggered. BCR auto-approves markup. |

BCR CERTIFICATION + CONTACT Certification Body BlackCat Robotics Field Operations Contact (346) 256-5851 Email blackcatrobotics.ai@gmail.com Platform dashboard.blackcatrobotics.com

© 2026 BlackCat Robotics · blackcatrobotics.com · All rights reserved.